# Merrymeeting Bay Volunteer Water Quality Monitoring Program: An Update

Printed in the Fall 2005 issues of Merrymeeting News (without graphs)

Background

Once a month, from April to October, volunteers all across the Bay put on their waders and collect water samples as part of our Volunteer Water Quality Monitoring Program. These volunteers attend a training session each spring where they learn the correct methods for determining pH, turbidity and dissolved oxygen, as well as for collecting water and air temperatures, tidal information and general information about the state of their sampling site.

The data collected by volunteers provide an important source of information about the ever-changing health of Merrymeeting Bay.  Friends of Merrymeeting Bay’s annual data collection supplements data collected by the Maine Department of Environmental Protection, who operate on a five year sampling regime, one year for each major watershed in Maine (Saco, Androscoggin, Kennebec, Penobscot and St. John).  The sampling efforts of volunteers have not gone unnoticed; in fact it was water quality data collected by Friends of Merrymeeting Bay that supported our successful reclassification proposal of the Kennebec River in 2003.

Rivers are given classification ratings based on their water quality. Due to years of pollution from industrial and municipal sources the Kennebec River had a rating of C. However, with the decline of pollution from paper mills and the removal of Edwards Dam, significant water quality improvements have occurred. The regular collection of water samples by Friends of Merrymeeting Bay volunteers documented the water quality improvements.  Specifically, dissolved oxygen data showed concentrations in the range of a Class B river--making a compelling case for reclassification. Reclassification is an important tool to protect a river because once a river is given a higher classification rating, it becomes illegal for it to backslide to a lower water quality. In other words, a higher classification means more stringent pollution limits govern future discharges into the river.

The Data (2003-2005)

Our sampling program has expanded since the Kennebec reclassification; we have sampling sites up and down the Kennebec and Androscoggin Rivers from Lines Island below the Chops to Norridgewock on the Kennebec and from the Bay to Greene on the Androscoggin.  Data from the past three years (2003-2005) were compiled and divided according to the location of the sampling sites; the Kennebec River, the Androscoggin River and the minor tributaries and Bay region. Seasonal trends were examined with specific interest in the pH, turbidity and dissolved oxygen. Each of these measures is explained below.

pH

pH is a measure of the acidity in water. The logarithmic pH scale ranges from 0-14. Healthy rivers in Maine have a pH between 6.5 and 8.5. The three-year average pH in each of our sampling areas is 6.7, 6.94, and 6.76 in the Androscoggin, Kennebec, and tributaries & Bay regions, respectively.  pH at all the sampling sites is in a healthy range, with only a few isolated peaks or dips over the three-year period, the pH in the Bay is also very stable showing little change with the seasons.

# Turbidity

Turbidity is a measure of the cloudiness in water. Turbidity can be caused by soil erosion, waste discharge, nutrient loading, urban runoff, bottom feeders like carp that stir up sediments, and algal growth.  Suspended solids in turbid water can clog fish gills, and prevent egg and larval development making turbidity an important indicator of the health of the Bay.  The turbidity along the Androscoggin and Kennebec are consistently low with respective three-year averages of 4.43 JTU (Jackson Turbidity Units) and 5.91 JTU.  Due to the heavy precipitation we received this past fall turbidity spikes were recorded in the Kennebec. October 2005 turbidity levels at sites from Abagadasset Point to Hallowell went from their normal range of 0-10 JTU range to that of 20-30 JTU.

The smaller tributaries flowing into the Bay such as the Cathance and Eastern River (at downstream locations) have much higher turbidity than any other sites sampled along the Bay.  Graphed levels [see graphs on our web site in the cybrary section] show higher turbidity readings at the Bowdoinham Boat Launch, the Eastern River at the Route 27 Bridge and the Abagadasset River at the Route 24 Bridge.  The reason for the higher turbidity in the tributaries is unknown, though possible causes include disturbance from carp and/or from a combination of soil erosion and human impact.

# Dissolved Oxygen

Dissolved oxygen measures the amount of gaseous oxygen (O2) dissolved in the water. Oxygen gets into water mainly by diffusion from the surrounding air, and as a by-product of photosynthesis. The amount of dissolved oxygen in a system is indicative of its ability to support aquatic life.  Dissolved oxygen all along the Bay follows a strong seasonal pattern where oxygen levels are lowest during the hot summer months and higher during spring and fall. The seasonal changes occur because cold water has a higher capacity to hold dissolved oxygen than warm water, and because lower flow rates in the summer means less water mixing with the air and a decrease in dissolved oxygen in the water.

Seasonal patterns of dissolved oxygen concentrations are evident at all the Kennebec River sites for the three years of collected data.

Fecal Coliform

Coliform bacteria are naturally occurring bacteria in water and soil; however one type of coliform bacteria, Escherichia coli (E.coli), is an indicator of fecal contamination. Fecal contamination in a water source can be caused by nearby wildlife, an overload to a wastewater treatment plant, or excessive runoff from non point sources. Fecal coliform are associated with gastro intestinal disorders and are a health hazard at certain levels of contamination. In fact, E. coli contamination is the most common reason for closing of shellfish harvesting areas.

Fecal coliform is the second criteria on which Maine river classifications are based (dissolved oxygen being the other). Therefore, in the spring of 2005 FOMB began a pilot program for sampling fecal coliform bacteria at five locations around the Bay--both monthly and after rain events. Maine law requires that a class B River not exceed a geometric average fecal level of 64 bacteria per 100 milliliters (ml) of water and an instantaneous level of 236 per 100 ml of water during the summer months.

For the summer months, fecal coliform levels in the Bay ranged from 0-740 per 100 ml during the monthly baseline sampling and 0-340 per 100 ml during rain events. We saw no evidence this summer of changes in fecal coliform counts due to increased precipitation. The geometric average for the Bay from May- August 2005 was 15.66 fecal coliform per 100 ml, which falls within the class B requirements.  From May to August the highest single count of bacteria was 220; this was after a rain event and does not exceed the instantaneous level specified in the Class B requirements.

During a large precipitation event in October fecal coliform counts in the Bay reached the highest levels of the season with an average of 1790 bacteria per 100 ml.  Given the amount of precipitation that fell in such a short amount of time these numbers do not seem unusual.

In order to ensure the waters of the Bay are healthy enough for fishing and swimming, continued sampling for fecal coliform on a regular basis with strict adherence to quality control measures is crucial.  The upgrade to the Kennebec River is a huge step towards ensuring cleaner water!

Many thanks to our cadre of nearly 20 dedicated volunteer monitors, and to Bill Milam for coordinating our testing program.

By Heather Caron

Editor’s note: Thanks are also due to the author, Heather Caron, who worked with Bill Milam to establish our pilot testing program for fecal coliform.

Other graphs of the 2003-2005 data